# BIOS update for EN9600

| Model | Part No | New VGA BIOS |
| --- | --- | --- |
| EN9600GT/HTDI/512M/A | *-C1CK95-J0A8* | V62.94.4B.00.AS75 |
| EN9600GT OC GEAR/HTDI/512M/A | *-C1CL40-J0A1* | V62.94.4B.00.AS73 |

- Check your VGA card Part No according to the paster on your VGA card as below. If the part No. is *-C1CK95-J0A8* or *-C1CL40-J0A1*, please go on BIOS flashing procedure.

- Unzip the “updateBIOS_EN9600.rar” and put it at root path. Such as C:\

- Left only one card when flash VGA BIOS. The flash tool can’t flash BIOS if you plug two cards at same time.

- Run “updateBIOS.exe”, press "Y" when the program promote.

- Reboot system when BIOS flash has been finished.

- If you can’t flash VGA BIOS on certain motherboard which support Hybrid SLI/Crossfire, please update your motherboard BIOS first.

- If you can’t flash VGA BIOS with certain motherboard, please change PCI-E slot for try. We found that you need to change to the black PCIE slot when flash VGA BIOS.