# DES-3500 Series Firmware Release Note

Firmware: 5.01.B52

Hardware:

DES-3526/26DC:A1/A2/A3/A4G

DES-3550:A1/A2/A3/A4G /A5G

Date: Oct.15, 2008

New features :

1. IP-MAC-Port Binding v3.3

2. DHCP Relay option 60/61

3. D-View Module support HP OV NNM v7.51

4. 802.1Q-2005(STP Root Restriction)

5. Configurable auto MDI/MDIX function

6. Gratuitous ARP

7. Cable Diagnostics

8. "Show tech support" command

9. 802.1x sesssion timeout attribute

10. VLAN batch configuration

11. Time-based ACL

12. Add RFC2869 RADIUS extension standard for two attribute "Acct-Input-Gigawords & Acct-Output-Gigawords"

13. Password encryption

14. 802.1X Port-Based Access Control with Target VLAN behavior (Add ingress/egress bandwidth attribute and 802.1p priority attribute)

15. Multicast router traffic control

16. BPDU restriction

17. S/N display on UI

18. Keep static VLAN setting and IPIF while resetting system

19. DHCP local relay

20. ARP Spoofing Prevention

Note:

- While upgrading FW from R5 to R5.1, the old user account existed in R5 FW can not be ported into configuration because R5 FW doesn’t support password encryption. In other words, user can not see old user account in configuration of R5.1.

- While downloading R5.1 configuration file, the originally existed user account will still be kept and won’t be cleared.

- While downgrading FW from R5.1 to previous release, all the configurations will be reset to default.

- The code size of R5.1 is bigger than R5, while setting dual images (especially with 2 sets of R5.1 FW), the device may not boot up successfully. It’s because PROM waste more time to check FW in flash so it can not notify the Watch Dog to set its timer in time. In order to solve this issue, we’ve modified the prom code to extend the FW checking duration and embedded the prom code in R5.1 runtime FW to upgrade prom code automatically at next boot up. It’s strongly recommend that user should follow the R5.1 FW upgrade procedure SOP to download R5.1 FW, otherwise user maybe have a fatal damage that need to RMA.