DP83820 GigMAC NetWare Release Notes

Release 1.0

10/23/2000

Read This Document Before Attempting To Install Or Use This Product!

This document contains information about factors that must be considered before, during, and after installation.

General Notice:

Other brand and product names used herein are for identification purposes only and may be trademarks of their respective companies.

# Table of Contents

Table of Contents	2

1.0 Introduction	3

2.0 Product Overview	3

3.0 Installation	3

3.1 Release Media	3

3.2 Installation Procedure	3

3.2.1 Hardware Installation	4

3.2.2 Software Installation and Configuration	5

3.2.3 Installation Troubleshooting	6

3.2.4 Disk Space Requirements	6

3.2.5 Errata	6

4.0 Product Documentation	7	Error! Bookmark not defined.

5.0 Problem Reporting	7

National Semiconductor DP83820 Gigabit Ethernet Adapter

# 1.0 Introduction

This document presents information to users about National Semiconductor’s DP83820 Gigabit Ethernet adapter and the relevant NETWARE software driver version 1.0 for NetWare operating system version 5.1.

# 2.0 Product Overview

The DP83820 is a PCI Gigabit Ethernet adapter from National Semiconductor which supports a 33/66MHz, 32/64 bit PCI 2.1/2.2 bus interface. It complies with the emerging 1000 Mbps Ethernet 802.3z specification and supports full duplex operation.

Other features include:

- Maximized PCI efficiency and performance

- Pause Frames

- Checksum Offloading

- Jumbo Frames

- Wake-on-LAN

- VLAN support

The driver for this adapter on NetWare is designed to be a NIC driver for processing Ethernet data packets.

# 3.0 Installation

## 3.1 Release Media

The release media consists of the National Semiconductor’s DP83820 GigMAC NetWare driver which is available on disk.

## 3.2 Installation Procedure

This section describes the installation of the adapter and the driver software for making the adapter functional on the Windows platforms.

### 3.2.1 Hardware Installation

This section describes the installation of the GigMAC demo card into a

personal computer (PC). To install the card, you need:

• the GigMAC demo card

• a PC with an available PCI slot

• screwdrivers to open the PC and secure the GigMAC demo card

• a Category 5 twisted-pair RJ-45 cable

• a wrist strap

• a blank, formatted floppy disk

Installing the card consists of one optional and three required

stages: Copying the Driver to a Floppy Disk (optional), Inserting

the Card, Connecting the Network, and Installing the Driver. If the

PC into which you are installing the GigMAC demo card is

equipped with a CD-ROM, you may skip the first stage and begin

with stage II.

Inserting the Card

The second stage in the installation sequence is inserting the card

into the PC. For this stage you will need the GigMAC demo card and the

screwdrivers. Unplug the PC before opening it to avoid electrocution.

Be sure to ground yourself before handling the GigMAC

card to avoid ElectroStatic Discharge (ESD) damage to the card.

1. Shutdown the PC, and disconnect the power cord.

2. Open the chassis of the PC.

3. Choose an unused PCI slot and remove its metal bracket by

loosening the screw on the inside. You will no longer need this

metal bracket as the GigMAC demo card has its own. You will need

the screw, however.

4. Remove the GigMAC demo card from the ESD-safe packaging.

Warning: Static charge from your body can permanently damage

the GigMAC demo card. Do not handle the card without first

electrically grounding yourself via wrist strap or by touching a

large piece of metal.

5. Insert the GigMAC demo card into the empty PCI slot in the PC.

Orient the card so that the RJ-45 connector points out of the

computer (See Figure 2).

Warning: The card may fit backwards (with the RJ-45 pointing

in) into the PCI slot. Starting the computer with the card

inserted backwards may damage the card and/or the PC.

6. When you are certain the card is oriented correctly, press

firmly on the card to ensure that it is completely seated in the

PCI slot. Secure the GigMAC demo card using the screw from step

3.

7. Close the computer, and reconnect the power cord.

Connecting the Network

The third stage in the installation sequence is connecting the GigMAC

demo card to the network. In this stage, you will need only the

twisted-pair Category 5 network cable.

After you have inserted the card and closed the PC, connect one end

of the twisted-pair cable to the RJ-45 connector on the GigMAC  demo

card, and the other end to another PC or a network hub or switch

(see Figure 4). When connecting the card to another PC, the cable

will need to be crossover, meaning that pins 1 and 2 must be

swapped with pins 3 and 6, respectively, between the two ends of

the cable. A crossover cable should not be used when connecting

the GigMAC demo card to a network hub or switch. The difference

between a crossover and non-crossover (“straight”) cable is shown

The next time you turn on the PC, one of the two green link lights

on the GigMAC demo card should become illuminated almost immediately.

This signifies that the card has established link with the

device at the other end of the cable and is ready to send and receive

data.

### 3.2.2 Software Installation and Configuration

The fourth stage in the installation sequence is installing the driver

for the GigMAC demo card. The following subsections describe the

procedure for installing the driver for NETWARE.

NETWARE Driver Installation

Follow this sequence for installation under NETWARE.

Install the NIC into a PCI slot of a NetWare server and note the slot number for driver configuration later (slots are generally numbered top/front to bottom/back of the machine). It is recommended that NetWare 5.1 with Support Pack 1 or later be installed. To install the driver, copy the dp83820.lan & dp83820.ldi files onto a floppy disk.

Load NWConfig.nlm and follow the 'Driver Options->Configure Network drivers->Select a driver' options on the UI. Follow on-screen directions to point to the drivers installed on floppy (A:). Put the slot number and the number of Tx, and Rx Descriptors as required.

Alternately, driver can be loaded from the command line (e.g. load a:\dp83820.lan) & passing the required parameters. This will display the available slot options for the NIC.

The DP83820.LAN may also be configured at OS install time and can be used in conjunction with the auto-detect feature on NetWare Install.

The different options for speed and duplex mode can be configured using NWConfig at install

time or Inetcfg later.

### HOT PLUG

-----------

Use NcmCon.nlm from Novell & refer to the System vendor's documentation for the systems which support hot-plug functionality.

### 3.2.3 Installation Troubleshooting

### 3.2.4 Disk Space Requirements

### 3.2.5 Errata

This section describes the features and issues to be provided upon final release of the driver.

- May not load on NW 4.x.

- Associated logs represent testing in 32-bit slots, NetWare5.1 and single processor systems only.

- Wake on LAN / Power Management Support

NETWARE driver architecture specifies no specific support required for Power Management.

TCP/IP Checksum Offload Support

Although the DP83820 chip does support checksum offload, the beta release of this driver does NOT support checksum offload.

VLAN (802.1Q) & QoS (802.1P) Support

NETWARE driver architecture specifies no specific support required for VLAN and QoS.

# 4.0 Product Documentation

This release notes document, National Semiconductor Corp DP83820 Gigabit Ethernet Adapter Driver Release Notes, provides detailed information about installing the National Semiconductor Corp DP83820 Gigabit Ethernet Adapter and Driver software.

# 5.0 Problem Reporting

National Semiconductor Corporation

2900 Semiconductor Drive

Santa Clara, CA 95051, U.S.A.

Tel: 1-800-272-9959

Fax: 1-800-737-7018

Email: support@nsc.com

WWW: www.national.com

National Semiconductor Europe

Fax: (+49) 0-180-530 85 86

Email: europe.support@nsc.com

Deutsch Tel: (+49) 0-180-530 85 85

English Tel: (+49) 0-180-532 78 32

National Semiconductor Asia Pacific

Customer Response Group

Tel: 65-254-4466

Fax: 65-250-4466

Email: sea.support@nsc.com

National Semiconductor Japan Ltd.

Tel: 81-3-5620-6175

Fax: 81-3-5620-6179