# Update U.S. Robotics Wireless Access Point Firmware

This package includes:

- Readme file

- HTML .img file

- Firmware .img file

- Open the U.S. Robotics Wireless Access Point Configuration Utility.  Scan for the Access Point you wish to upgrade by going to the AP drop down menu and select scan.

- Disconnect all PC’s and Laptop Computers from that Access Point except the one from which you will do the update.  Do this either by shutting them down or by moving the Access Point to an isolated network.  (It is not recommended to have Wireless Clients talking to the ACCESS POINT while the firmware is being updated.)

- To verify that there is only one workstation connected to the access point, double click on the access point.  This will open the web interfaced configuration site.  Type your user name and password (The default configurations are:  User Name: default; Password: WLAN_AP <All Caps>).  Click on Information MIB.  Check to verify that the association number is 1 if you are connecting via wireless or 0 if you are connecting via wired Ethernet.

- Highlight the Access Point.  Then Click on the AP drop down menu.  Click on UPDATE FW (Firmware).

- After clicking on it, a dialog box will pop up and request USERNAME, PASSWORD and .img file.  (The default configurations are:  User Name: default; Password: WLAN_AP <All Caps>).  Click on UPDATE to begin the flash process.

- The Access Point has 2 flash ROM chips on it.  One chip contains the information on the firmware and the other contains the HTML Graphic Interface.  This package contains 2 .img files for update.  The previous 5 steps will need to be completed twice (once for each flash .img) to successfully update the ACCESS POINT to the most recent versions of firmware.